James Stallkamp

ECE 375 HW1

1 Explain the following.

- Opcode: An opcode is a string that specifies the operation to be performed.

- Operand: An operand is the piece of an instruction that specifies the data to be operated on

- Effective Address: The effective address is a part of an instruction that acts like a pointer and refers to the actual operand in memory.

- Program Counter: The program counter is essentially a line counter, it keeps track of where in the code execution is currently at, it is used to fetch instructions, data, and more.

2 CPU with I address format and memory of 1k(1024). Supports 32 instructions. Instructions has three parts, opcode, indirect mode bit, and address field. If I-bit =0, the operand is located in M[Address],else the operand located in M[M[Address]]

Draw the instruction word format indicating the number of bits and the function of each field.

| Opcode:5 bits | I – Bit: 1 bit | Address: 10 bits |
| --- | --- | --- |
| 2^5=32, need to support 32 opcodes | Only needs to represent 1 or 0 | 2^10 = 1024, need that many bits to address all of the memory space |

3 Draw a block diagram of the hardware to connect C to the internal data bus and the ALU in the provided CPU, using a tri state buffer and a multiplexer.

MUX

Internal Data Bus

ALU

A

B

Result

1

0

AC

Enable

4 Explain whether or not each of the following microoperations can be performed in a single clock cycle. PC and MAR each contain 12 bits and AC and MDR each contain 16bits and IR is 4 bits

- MAR ← AC, MDR ←MAR. No you can’t simultaneously write the contents of MAR and read the contents of MAR in order to write them elsewhere.

- IR← MDR, MAR ←MDR. Yes. Parallel write from MDR to IR and MAR.

- MAR ←MDR,MDR←M(MAR). No, reading and writing to MDR simultaneously.

- MDR←AC+1. Yes, AC can increment itself and be read.

- AC←MDR,PC←PC+1. Yes two operations are separate and PC can increment itself

- PC←PC+AC. No? Program counter cannot do arithmetic with the accumulator.

5 Give the microoperations required to implement the execute phase of STA (X)+. Given generic pseudo cpu from book. 16bit inst, 4bit opcode,12bit address. Operands are 16bits. PC and MAR are 12bits. AC,MDR,TEMP each contain 16bits. IR is 4 bits.

GIVEN FETCH

- MAR←PC

- MDR←M(MAR),PC←PC+1

- IR←MDR(Opcode),MAR←MDR(address)

Execute

  - MDR←M(MAR),TEMP←AC

  - AC←MDR

  - AC←AC+1

  - MDR←AC

  - M(MAR) ←MDR,AC←AC-1

  - MAR←AC

  - MDR←TEMP,AC←TEMP

  - M(MAR) ←MDR

6

- Ldi r27,85

| register | contents |
| --- | --- |
| x | 0106->8506 |

Loading x high with 85

- Ror r2

| register | contents |
| --- | --- |
| 2 | 1b->8D |

Performing a rotate right through carry on r2, r2 is holding 1b-> 00011011, rotated: 10001101->8D

- Adc r2,r1

| register | contents |
| --- | --- |
| 2 | 1b->20 |

Performs add with carry, result in r2. 1b+05(hex) =27+5(dec)=32(dec)=20(hex)

- Sts $0007,r28

| Data Memory address | contents |
| --- | --- |
| $0107 | 02->02 |

writes the value in YL into memory address $0007. Seems like I made a mistake somewhere because the handout does not show a memory location $0007. But I feel confident that STS $XXXX Rs, is supposed to write the contents of register Rs to memory location $XXXX.

- Sbiw XH:XL, 2

| Register | contents |
| --- | --- |
| x | 0106->0104 |